Silicon  4nm

# (c) Silvaco Inc., 2019

go atlas

# Sweep increment

set vstart = 0

set vstop = 25

set vinc = 1

mesh width=100

# x plane meshing

x.m l=0.0 s=0.25

x.m l=0.5 s=0.25

x.m l=1.0 s=0.25

x.m l=3.5 s=0.15

x.m l=5.5 s=0.15

x.m l=8   s=0.15

x.m l=8.5 s=0.25

x.m l=9   s=0.25

# y plane meshing

y.m l=0 s=0.25

y.m l=0.496 s=0.05

y.m l=0.5 s=0.001

y.m l=0.525 s=0.001

y.m l=0.6 s=0.05

y.m l=0.7 s=0.1

y.m l=1.0 s=0.2

y.m l=2.0 s=1

region num=1 x.min=0 x.max=9 y.min=0.5 y.max=0.525 mat=AlGaN x.comp=0.3

region num=2 x.min=0 x.max=9 y.min=0.0 y.max=0.496 mat=nitride insulator

region num=3 x.min=0 x.max=9 y.min=0.525 y.max=2 mat=GaN  substrate

region num=4 x.min=0 x.max=9 y.min=0.496 y.max=0.5 mat=SiO2  oxidelayer

elec num=1 name=source x.min=0 x.max=0.5 y.min=0.5 y.max=0.6

elec num=2 name=drain x.min=8.5 x.max=9 y.min=0.5 y.max=0.6

elec num=3 name=gate x.min=3.5 x.max=5.5 y.min=0 y.max=0.496

elec num=4 substrate

contact name=gate work=5

contact name=source work=3.93

contact name=drain work=3.93

models print srh

mobility albrct.n

model polarization calc.strain polar.scale=0.8

output con.band val.band charge  polar.charge band.par

method newton trap maxtrap=20

solve init

save outf=ganfetex03_0.str

solve vdrain=1.0

solve name=gate vfinal=-15 vstep=-1

log outf=ganfetex03_0.log

solve name=gate vfinal=1.0 vstep=0.5

log off

solve init

solve vgate=-6

log outf=ganfetex03_1.log

solve name=drain vdrain=$vstart vfinal=$vstop vstep=$vinc

log off

#

solve init

solve vgate=-4

log outf=ganfetex03_2.log

solve name=drain vdrain=$vstart vfinal=$vstop vstep=$vinc

log off

#

solve init

solve vgate=-2

log outf=ganfetex03_3.log

solve name=drain vdrain=$vstart vfinal=$vstop vstep=$vinc

log off

#

solve init

solve vgate=0

log outf=ganfetex03_4.log

solve name=drain vdrain=$vstart vfinal=$vstop vstep=$vinc

log off

# Apply a set of biases at the gate and drain

solve init

solve vgate=-2

solve vdrain=10.0

# Frequency domain AC simulation.

log outf=ganfetex03_ac.log master gains s.params inport=gate outport=drain width=100

solve ac freq=10 fstep=10 mult.f nfstep=7

solve ac freq=1e10

solve ac freq=2e9 fstep=2e9 nfstep=4

solve ac freq=1e10 fstep=5e9 nfstep=10

tonyplot ganfetex03_0.log -set ganfetex03_1.set

tonyplot -overlay ganfetex03_1.log ganfetex03_2.log ganfetex03_3.log ganfetex03_4.log -set ganfetex03_2.set

tonyplot -overlay ganfetex03_1_lat.log ganfetex03_2_lat.log ganfetex03_3_lat.log ganfetex03_4_lat.log -set ganfetex03_2.set

tonyplot ganfetex03_0.str -set ganfetex03_3.set

tonyplot ganfetex03_ac.log -set ganfetex03_2.set

tonyplot ganfetex03_ac.log -set ganfetex03_3.set

log off

#go atlas

#mesh inf=ganfetex03_0.str width=100

#region modify num=3 substrate

#contact name=gate work=5

#contact name=source work=3.93

#contact name=drain work=3.93

#models print srh

#mobility albrct.n

#model polarization calc.strain polar.scale=0.8

#model lat.temp

# the following statements are for thermal modeling

#thermcontact num=1 y.min=2 y.max=2 ext.temp=300

#

#output con.band val.band charge  polar.charge band.par

#

#method block newton

#

#solve init

#solve vgate=-3

#log outf=ganfetex03_1_lat.log

#solve name=drain vdrain=$vstart vfinal=$vstop vstep=$vinc

#log off

##

#solve init

#solve vgate=-2

#log outf=ganfetex03_2_lat.log

#solve name=drain vdrain=$vstart vfinal=$vstop vstep=$vinc

#log off

##

#solve init

#solve vgate=-1

#log outf=ganfetex03_3_lat.log

#solve name=drain vdrain=$vstart vfinal=$vstop vstep=$vinc

#log off

##

#solve init

#solve vgate=0

#log outf=ganfetex03_4_lat.log

#solve name=drain vdrain=$vstart vfinal=$vstop vstep=$vinc

#log off

Quit